Arduino-Based Object Detection, Tracking

& Alert System

Session: 2024 – 2028

Submitted by:

Muhammad Ali                                2024-CS-25

Zaid Salman                                     2024-CS-42

Ahsan Subhani                                 2024-CS-43

Numair                                             2024-CS-19

Kamran Sattar                                   2024-CS-30

Submitted To:

Sir Kamran Mustafa

Department of Computer Science

University of Engineering and Technology

# Abstract

This project presents a low-cost, Arduino-based object detection and tracking system utilizing an ultrasonic sensor, servo motor, laser module, and active buzzer. The system continuously scans the environment, detects objects within a defined threshold range, and responds by pointing toward the target using the servo motor while activating visual (laser) and audio (buzzer) alerts. The project successfully simulates a simplified radar like mechanism, making it highly suitable for educational robotics, embedded systems learning, and foundational surveillance prototyping.

Contents

Abstract	1

1 Introduction	4

1.1 Project Overview	4

1.2 Objectives	4

2 Hardware Design & Connections	5

2.1 Components Used	5

2.2 Circuit Connections	5

3 System Architecture & Operation	7

3.1 Working Principle	7

3.2 System Behavior Matrix	7

3.3 Flow Diagram	8

4 Software Implementation	9

4.1 Source Code Summary	9

4.2 Code Logic & Explanations	9

4.2.1 Distance Calculation Formula	9

4.2.2 Detection Condition	9

4.2.3 Servo Scanning Logic	9

5 Challenges & Safety Considerations	10

5.1 Challenges Faced	10

5.2 Safety Considerations	10

6 Conclusion & Future Scope	11

6.1 Conclusion	11

6.2 Future Improvements	11

List of Tables

| 2.1 List of Components and their Purpose. . . . . . . . . . . . . . . . . . . | 5 |
| --- | --- |
| 2.2 Ultrasonic Sensor (HC-SR04) Connections . . . . . . . . . . . . . . . . . | 5 |
| 2.3 Actuator and Alert Module Connections . . . . . . . . . . . . . . . . . . | 6 |
| 3.1 System Behavior Matrix . . . . . . . . . . . . . . . . . . . . . . . . . . . | 7 |
| 5.1 Troubleshooting and Solutions . . . . . . . . . . . . . . . . . . . . . . . . | 10 |

List of Figures

3.1	Logical Flow of Operation . . . . . . . . . . . . . . . . . . . . . . . . . .	8

# 1. Introduction

## 1.1 Project Overview

The integration of embedded systems in object tracking and detection plays a crucial role in modern robotics, automation, and security systems. This project mimics the fundamental principles of a radar system by employing an ultrasonic sensor mounted on a 180-degree sweeping servo motor.

## 1.2 Objectives

The primary objectives of this project are as follows:

- To design and implement a real-time object detection system.

- To implement servo-based directional scanning.

- To accurately detect objects using an HC-SR04 ultrasonic sensor.

- To trigger multi-modal alerts utilizing a buzzer and a laser module.

- To successfully simulate radar-like motion behavior and target locking.

# 2. Hardware Design & Connections

## 2.1 Components Used

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino Uno | 1 | Main microcontroller processing logic. |
| Ultrasonic Sensor (HC-SR04) | 1 | Distance measurement and object detection. |
| Servo Motor (SG90) | 1 | Facilitates directional scanning. |
| Laser Module | 1 | Serves as a visual alert indicator. |
| Active Buzzer | 1 | Provides an audio alert upon detection. |
| Breadboard | 1 | Central platform for circuit assembly. |
| Jumper Wires | Multiple | Establishes electrical connections. |
| External 5V Supply | 1 | Ensures servo power stability (Optional but recommended). |

The system relies on readily available, low-cost electronic components centered around the Arduino microcontroller.

Table 2.1: List of Components and their Purpose

## 2.2 Circuit Connections

Proper wiring is essential for the stable operation of both the sensors and actuators. Tables 2.2 through 2.3 detail the pin configurations.

| Sensor Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | Pin 10 |
| ECHO | Pin 11 |

Table 2.2: Ultrasonic Sensor (HC-SR04) Connections

| Module | Module Pin | Arduino / Power Pin |
| --- | --- | --- |
| Servo Motor | Signal VCC | Pin 12 5V (External Recommended) |
|  | GND | GND |
| Active Buzzer | Positive (+) Negative (-) | Pin 5 GND |
| Laser Module | Positive (+) Negative (-) | Pin 6 GND |

Table 2.3: Actuator and Alert Module Connections

# 3. System Architecture & Operation

## 3.1 Working Principle

The core operational logic relies on a continuous mechanical sweep and acoustic ranging:

- The servo motor sweeps continuously from 0◦ to 180◦ and back.

- At every incremental angle, the ultrasonic sensor fires a sound pulse to measure distance.

- If an object is detected within the defined threshold (≤ 30 cm), the servo halts at the current angle, locking onto the target. The Arduino simultaneously activates the laser and buzzer.

- If the object moves out of range or is removed, the system deactivates the alerts and resumes scanning.

## 3.2 System Behavior Matrix

The state of the actuators varies strictly based on the distance feedback from the sensor, as outlined in Table 3.1.

| Environmental Condition | Servo State | Laser State | Buzzer State |
| --- | --- | --- | --- |
| No object in range | Scanning (0◦ − 180◦) | OFF | OFF |
| Object detected (≤ 30cm) | Stopped (Target Lock) | ON | ON |
| Object lost | Resume Scanning | OFF | OFF |

Table 3.1: System Behavior Matrix

## 3.3  Flow Diagram

Figure 3.1 illustrates the logical execution flow programmed into the Arduino.

Start

InitializeSensors&Servo

Scan0

◦

→

180

◦

MeasureDistance

Distance

≤

cm?

30

StopServo

TurnONLaser&Buzzer

ContinueScanning

Yes

No

Figure3.1:LogicalFlowofOperation

# 4. Software Implementation

## 4.1 Source Code Summary

The embedded C++ code written in the Arduino IDE manages hardware interfacing. The main functional logic entails reading acoustic durations, calculating spatial distance, mapping angles to the servo, and toggling GPIO pins based on conditional statements.

## 4.2 Code Logic & Explanations

### 4.2.1 Distance Calculation Formula

To determine the distance of an object, the duration of the ultrasonic echo is multiplied by the speed of sound.

distance = duration * 0.034 / 2;

Explanation: The speed of sound in air is approximately 343 m/s, or 0.034 cm/µs. The division by 2 accounts for the sound wave traveling to the object and reflecting back.

### 4.2.2 Detection Condition

if (distance > 2 && distance < 30) { // Trigger Alerts }

Explanation: This condition filters out hardware noise (distances under 2 cm, which HCSR04 struggles to measure accurately) and sets an upper limit of 30 cm for positive target identification.

### 4.2.3 Servo Scanning Logic

angle += step; servo.write(angle);

Explanation: Increments the servo position by a defined step, creating a smooth, radarlike mechanical sweep across the operational plane.

## Project Resources & Media

All project resources including the Arduino source code, wiring schematics, and hardware images have been made publicly available on GitHub for reference and replication:

https://github.com/AhsanSubhani3363/COAL-Final-Project-

# Challenges & Safety Considerations

## 5.1 Challenges Faced

During the prototyping phase, several integration issues were encountered and resolved.

| Problem Encountered | Implemented Solution |
| --- | --- |
| Servo motor stuttering/not moving | Fixed by implementing a dedicated external 5V power supply to prevent current draw issues on the Arduino. |
| Buzzer failing to emit sound | Resolved by verifying the correct GPIO pin assignment and utilizing the tone() function in code. |
| Unstable ultrasonic readings | Reduced the maximum detection threshold and added a lower limit to filter out electronic noise. |
| Erratic system behavior | Ensured a common ground connection between all modules, external power, and the Arduino. |

Table 5.1: Troubleshooting and Solutions

## 5.2 Safety Considerations

When replicating or operating this system, the following safety protocols must be observed:

- Laser Safety: A low-power laser module is used. Direct eye exposure must be avoided to prevent retinal damage.

- Electrical Grounding: Ensure all grounds (GND) are tied together to prevent floating voltages and component damage.

- Power Management: Do not draw excessive current directly from the Arduino 5V pin for the servo; an external power source is highly recommended.

# Conclusion & Future Scope

## 6.1 Conclusion

This project successfully demonstrates the design and execution of an automated, radarlike object detection and tracking system using an Arduino microcontroller. By integrating an HC-SR04 ultrasonic sensor with a servo motor, the prototype achieves real-time environmental scanning. The addition of a laser and active buzzer provides immediate, intuitive feedback upon target locking. The system is reliable, highly scalable, and serves as an excellent foundation for understanding closed-loop sensor-actuator relationships in robotics.

## 6.2 Future Improvements

While the current prototype meets all primary objectives, the system architecture can be upgraded for advanced applications:

- Multi-Sensor Array: Implementing multiple ultrasonic or infrared sensors for 360-degree, true-direction detection.

- Visual Interface: Adding an LCD or OLED display for real-time distance and angle visualization.

- Radar UI Integration: Interfacing the Arduino with a PC using Processing software to create a graphical radar screen.

- Wireless Control: Incorporating a Bluetooth (HC-05) or Wi-Fi (ESP8266) module for remote monitoring and manual override control.

- AI & Vision: Replacing the ultrasonic sensor with a camera module (e.g., ESP32CAM) running OpenCV for AI-based object classification (distinguishing between humans, animals, and vehicles).